| 1. GENERAL INFORMATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name of project | Rebecca’s Mortality Burial Pit |  |  |  |  |
| Name of person responsible | Rebecca Aucoin |  |  |  |  |
| Address | 123 Chicken House Road, Farm Township, PA 15555 |  |  |  |  |
| Phone number | 717-555-5555 |  |  |  |  |
| Municipality where project is located | Farm Township |  |  |  |  |
| Attach a general location map | See attached E&S Plan |  |  |  |  |
| Have you contacted the municipality | YES NO |  | NOTE: MUNICIPAL PERMITS MAY BE REQUIRED |  |  |
| Have any naturally occurring geologic formations or soil conditions that may have the potential to cause pollution during earth disturbance activities been identified onsite? YES NO If yes, explain: |  |  |  |  |  |
| Brief description of project Project involves excavation of a 100’ x 40’ pit for burial of 36,000 broiler chickens. The pit will be excavated to a depth of 4 feet. All excavated soil will be placed on the downhill side of the pit. The bottom of the pit will be at least 2 feet above bedrock and the seasonal high water table. The pit will then be covered with a minimum of 2 feet of topsoil within 48 hours and covered with seed and mulch. |  |  |  |  |  |
| Estimated start and end dates | Start Date | 7/15/15 |  | End date | 7/17/15 |
| Name of nearest receiving stream | Bird Run |  |  | Distance | 160 Feet |

Erosion and Sedimentation Pollution Control (ESPC) Plan Narrative

| 2. SOILS INFORMATION |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SOIL NAME Calvin-Leck Kill (CLB2) | SLOPE | 5% | DEPTH | 6 feet | IS SOIL HYDRIC | YES NO |
| LIMITATIONS unstable excavation walls, slope, depth to bedrock (check depth to bedrock prior to excavation of pit) |  |  |  |  |  |  |
| SOIL NAME Calvin-Leck Kill (CLC2) | SLOPE | 9% | DEPTH | 6 feet | IS SOIL HYDRIC | YES NO |
| LIMITATIONS unstable excavation walls, slope, depth to bedrock (check depth to bedrock prior to excavation of pit) |  |  |  |  |  |  |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |

Note 1 – Soils can be found at the Web Soil Survey ( http://websoilsurvey.nrcs.usda.gov/app/)

Note 2 - Hydric soils may indicate the presence of wetlands and that these areas should be avoided.  If not, permits from PADEP and/or US Army Corps of Engineers may be required

3. Typical Construction Sequence

In order for an ESPC plan to be effective, construction must take place in an organized sequence that integrates the ESPC Best Management Practices (BMPs) into the sequence in a timely manner. The first step in nearly all projects is the installation of sediment barriers below the project, that is downslope and installation of BMPs to control off site runoff that drains to the site. The sequence would then include other needed BMPs and end with the removal of temporary controls. A typical construction sequence is provided below.

A. Contacted Farm Township to see if I needed any necessary permits

B. Selected appropriate area for burial (see regulations and the cover sheet attached for minimum standard and setback requirements)

C. Install perimeter controls 1. Silt Fence & Rock Construction Entrance at locations shown. 2. Install and immediately stabilize vegetated diversion swale. 3. Rock Filter Outlet at end of diversion swale.

D. Excavate burial area

E. Backfill burial area with a minimum of 2 feet of soil within 48 hours

F. Seed and Mulch all disturbed areas according to DEP’s Erosion Control Manual

G. Ensure all disturbed areas achieve a minimum 70% uniform vegetative cover

| 4. TEMPORARY CONTROLS |  |  |  |
| --- | --- | --- | --- |
| Check the temporary controls that will be implemented for your project. Attach a drawing or design for any practice that is not listed in this guide. |  |  |  |
| Vegetated stabilization | x | Vegetated swale | x |
| Vegetated filter strip | x | Rock lined Swale |  |
| Silt fence | x | Rock filter | x |
| Filter sock |  | Rock outlet protection |  |
| Straw bales |  | Sediment trap |  |
| Rock construction entrance | x | Inlet filter bag |  |
| Other |  |  |  |
| Other |  |  |  |
| Other |  |  |  |

| 5. THERMAL IMPACTS |  |  |  |
| --- | --- | --- | --- |
| Identify BMPs to avoid, minimize or mitigate potential pollution from thermal impacts: |  |  |  |
| Vegetation/Vegetated Filter Strip | x | Locate project away from Waters of the Commonwealth | x |
| Limit Extent / Duration of Earth Disturbance | x | Avoid direct discharge to Waters of the Commonwealth | x |
| Other: |  | Other: |  |

6. PERMANENT CONTROLS

When a project is completed, it is necessary to ensure permanent stabilization of the site. This entails the stabilization of all exposed soil areas through revegetation, stone, pavement or other stabilization practice. List the permanent stabilization BMPs to be used below. Permanent stabilization BMPs should follow appropriate recommendations.

All disturbed areas will be seeded and mulched immediately after backfill is complete. All topsoil will be spread evenly

across the disturbed areas. The site will be inspected regularly to ensure proper stabilization has been achieved.

| 6. MAINTENANCE PROGRAM |  |
| --- | --- |
| All ESPC BMPs must be properly maintained in order to ensure that they function properly. List maintenance measures to be implemented below. The first four are general measures and are required. Common maintenance practices are also listed. Check those that apply. For maintenance practices not listed, describe the practice in the blank lines below and check. |  |
| Until final site stabilization, all ESPC BMPs will be properly inspected weekly and after each rainfall | X |
| All maintenance for ESPC BMPs will be conducted according to the recommendations in this guide | X |
| If any ESPC practice proves to be inadequate, additional measures will be immediately implemented | X |
| Upon final stabilization, all temporary ESPC BMPs will be removed | X |
| Sediment will be removed from silt fence when it accumulates to one half the height of the fence | x |
| Sediment will be removed from silt sock when it accumulates to one half the height of the silt sock |  |
| Sediment will be removed from straw bales when it accumulates to one third the height of the straw bale |  |
| Rock construction entrances will be maintained to adequate stone depth | x |
| Rock construction entrances will be cleaned out if clogged | x |
| Sediment will be removed from rock filters when it reaches one half the filter height | x |
| All accumulated sediment removed from any practice will be disposed of properly | x |
| Other: Any erosion forming in the diversion channel will be repaired immediately and re-stabilized. | x |

| 7. EROSION CONTROL PLAN DRAWING |  |  |  |
| --- | --- | --- | --- |
| A plan or drawing illustrating your ESPC plan is required. Section one is a blank sheet which may be used for this purpose. If you have a copy of a document showing your site, you may use that as well. Ensure the following, if applicable, are shown on the illustration in sufficient detail. |  |  |  |
| Name of project | x | Location and type of ESPC practice | x |
| North arrow | x | Property lines | x |
| Scale | x | Legend or key | x |
| Streams or ponds (include name if known) | x |  |  |
| Existing features such as buildings or roads | x |  |  |
| Proposed features | x |  |  |
| Direction of slopes | x |  |  |
| Soil types | x |  |  |

SECTION 1

ESPC PLAN DRAWING

|  |  | 540 | 538 | 2 foot contour lines |  | 536 | 534 |  | SAMPLE ESPC PLAN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | ClC2 ClB2 |  | 532 |  |  |  |  |
|  |  |  |  | DIVERSION CHANNEL (IF NEEDED) | PROPOSED BURIAL AREA | SOIL STOCKPILE |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | BIRD RUN |  | PROJECT NAME: CHICKEN BURIAL PIT_________ MAP KEY Property Line LIMIT OF DISTURBANCE CONSTRUCTION ENTRANCE FABRIC FILTER FENCE ROCK FILTER SWALE OUTLET TEMPORAY SWALE STREAM SOIL TYPE SCALE: 1 INCH = 40 FEET |  |  |
|  |  |  |  |  |  |  |  |  | CHICKEN HOUSE ROAD |  |  |  |  |  |  |

Vegetative stabilization

Vegetated stabilization is the use of vegetation to stabilize disturbed areas.  This includes areas where earth disturbance is final or temporarily completed.  Vegetation is commonly some type of grass that is planted on areas that have been disturbed.  The intent is to provide vegetative cover for disturbed areas as soon as possible after disturbance to protect exposed soil and reduce the potential for erosion of the soil.

AdvantageS:

-Relatively inexpensive

-Relatively simple

-Applicable on most situations

LimitationS:

-May be difficult to establish vegetation

during certain times of the year

-Soil amendments may be needed where

An example of vegetated stabilization - Vegetation established on this slope prevents erosion.  Courtesy of Dauphin County Conservation District

existing ground is not suitable for growth

Installation:

-Grass should be planted as soon as possible after disturbance is permanently or temporarily completed

-Topsoil should have a minimum depth of 4 to 8 inches

-Compacted topsoil should be loosened to a depth of 6-12 inches before seeding

-If topsoil needs to be added, the area should be loosened to a depth of 3-5 inches before applying topsoil

-If soil amendments are needed add amendments as indicated by a soil test or the Penn State Agronomy Guide.

-Seed mixtures should be selected based on the site factors such as:

amount of sunlight                    soil pH                    moisture                    slope                    expected use

-The seeded area should be mulched immediately after seeding

-On slopes of 8% or more mulch should be held in place with appropriate netting

-Erosion control blankets should be used where the site is close to surface water

There are a wide variety of methods for seeding and a wide variety of mulches, netting and control blankets available.  For projects where this guide is applicable, it is likely that simple seeding and mulching will be sufficient.  You may contact the Conservation District for more information.

SEDIMENT BARRIERS

Sediment barriers are typically installed at the perimeter of a site to prevent sediment from leaving the site.  These kinds of BMPs are referred to as perimeter controls.  There are many types of these controls available.  Three common types are discussed here.

FABRIC FENCE

Also called silt fence, is typically used to control sediment

from sheet flow on small project sites.  There are different

heights available for use, depending on site conditions.

Advantages:

-Relatively inexpensive

-Relatively simple

-Applicable on most situations

Limitations:

-Cannot be used where flow is concentrated

-There are limitations on maximum slope length

An example of filter fabric fence – This is a

30 inch reinforced fence.

From Erosion and Sediment Pollution Control program Manual, PADEP, March 2012

Courtesy of York County Conservation District                                                                           .

above the fence.  See slope/slope length Table below

and chart in section 6.

-Cannot be used in areas where the fence cannot

be completely anchored

-Cannot be used in uncompacted fills or loose soils

MAXIMUM SLOPE LENGTHS ABOVE FILTER FENCE BY TYPE

From Erosion and Sediment Pollution Control program Manual, PADEP, March 2012

| MAXIMUM SLOPE LENGTH, IN FEET ABOVE FENCE (see chart in Section 6 for greater detail) |  |  |  |
| --- | --- | --- | --- |
| Slope Percent | Standard (18” High) Silt Fence | Reinforced (30” High) Silt Fence | Super Silt Fence |
| 2 (or less) | 150 | 500 | 1000 |
| 5 | 100 | 250 | 550 |
| 10 | 50 | 150 | 325 |
| 15 | 35 | 100 | 215 |
| 20 | 25 | 70 | 175 |
| 25 | 20 | 55 | 135 |
| 30 | 15 | 45 | 100 |
| 35 | 15 | 40 | 85 |
| 40 | 15 | 35 | 75 |
| 45 | 10 | 30 | 60 |
| 50 | 10 | 25 | 50 |

Installation:

-Install perpendicular to slope on level grade to efficiently intercept surface flow

-Extend ends at least 8 feet upslope at 45 degree angles to prevent runoff from washing around the ends

-Fabric should be anchored firmly in a trench at least 6 inches deep to prevent underflow

-Backfill and compact the trench

-The ends of the fence should be wrapped around the end stakes and secured to the stake

-Fabric fence larger than 18 inches will require additional methods to secure the fence

Maintenance:

-Fence should be inspected on a regular basis and after each storm

-Remove accumulated sediment when the sediment reaches half the fence height

-If fence is undermined or topped, it should be repaired immediately.  This may also indicate the need for

other erosion control measures

STANDARD FILTER FABRIC FENCE INSTALLATION DIAGRAM

Filter fabric fence installation diagram – From Erosion and Sediment Pollution Control Program Manual, Pennsylvania Department of Environmental Protection, March 2012.

REINFORCED FILTER FABRIC FENCE INSTALLATION DIAGRAM

Reinforced filter fabric fence installation diagram – From Erosion and Sediment Pollution Control Program Manual, Pennsylvania Department of Environmental Protection, March 2012.

ROCK CONSTRUCTION ENTRANCE

When vehicles and equipment leave construction

sites, sediment is often carried with them on tires.

This sediment then washes into storm sewers or

road side ditches where it is ultimately delivered to

streams.   A rock construction entrance helps to

prevent the sediment from leaving the site by

removing sediment from tires.

An example of rock construction entrance - From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

Courtesy Lake County Stormwater Management Department, Ohio

ADVANTAGES:

-Relatively inexpensive BMP for vehicular

access to the site

-Effective at preventing most sediment

transport to adjacent roads or streets

LIMITATIONS:

-Not 100 percent effective.  Some sediment will still be transported from the site to roadways

-Must be maintained frequently

-Rock construction entrances are not used to remove sediment from runoff from areas above the entrance

INSTALLATION:

-Topsoil should be removed prior to installing the rock construction entrance

-Place geotextile over existing ground

-Rock should cover the full width of the entrance.  Twenty feet is the recommended minimum width

-If a culvert or pipe is needed to convey clean water, a berm is recommended to prevent crushing the pipe

Rock Construction Entrance Diagram

From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

MAINTENANCE:

-Entrance must be maintained to provide the specified depth of rock by adding rock when needed

-A stockpile of rock should be maintained on site for maintenance

-If the entrance becomes completely clogged, the entrance may need to be reinstalled

-Sediment that is deposited on roadways should be returned to the construction site

-Do not wash or sweep sediment on roadways, into gutters, ditches, inlets or other areas where it can be

washed into streams or storm sewers

SWALES

Swales are conveyance channels that collect and convey runoff.   For small sites, the typical use may be to capture and convey upslope runoff around disturbed areas.   Swales are also used to convey runoff from disturbed areas to sediment removal BMPs such as a sediment basin.  Swales can be lined with either vegetation or rock.

VEGETATED SWALE - Swales can intercept runoff and carry it safely around a project site.   Courtesy of Dauphin County Conservation District.

ROCK LINED SWALE - Swales carry runoff safely down

a steep slope.   From Erosion and Sediment Pollution

Control Program Manual, PADEP, March 2012

ADVANTAGES:

-Swales can prevent upslope runoff from reaching disturbed areas.   This reduces erosion and sediment transport.

-Swales may reduce or eliminate the need for additional controls

LIMITATIONS:

-Swale outlets may need additional stabilization

-Swales may not be practical on some sites

-Swale design may be complex

INSTALLATION:

Installation of swales is dependent on the design.  Consult the DEP publication “Erosion and Sediment Pollution Control Program Manual,  March 2012” for more information.  General installation includes:

- swales should be stabilized immediately after installation

- Unless the sod is to be installed, a fabric will be needed to stabilize the swale until the grass is established

MAINTENANCE:

-Inspect swales frequently

-Damage to swales should be immediately repaired

-Grass should be maintained at two to three inches

-Swales should be cleaned out if channel depth is reduced by 25%

ROCK FILTERS

Rock filters are rock barriers typically used in swales or channels to slow runoff.  This reduces the erosion potential of the flow and allows sediment to drop out.

ADVANTAGES:

-Rock filters can be used with swales to increase

effectiveness

-Relatively inexpensive and easy to construct

LIMITATIONS:

-Rock filters are not a substitute for

An example of a rock filter - Rock filters, such as the one above can be used to increase efficiency in swales.  From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

Courtesy of York County Conservation District

appropriate channel linings

-Rock filters may not be used where the channel

depth is less than two feet

INSTALLATION:

-The filter should be one half the channel depth

-Use R-3 rock for channels up to 3 feet deep

-Use R-4 rock for channels over 3 feet deep

-Construct the filter with a 6 inch depression in the center

-Place a 1 foot thick layer of small rock (AASHTO #57 or smaller) on the upstream side of the filter

Rock Filter Diagram

From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

MAINTENANCE:

-Inspect filters weekly and after each rain

-Repair filters immediately if needed

-If the filter is clogged, immediately replace the filter

-Sediment should be removed when it reaches one half the filter height